Home Automation via XBEE modules

2011-2012

Shayan Ali Bhatti & Shahbaz Hasan

Co-author : Sir Sajid Hussain

2011-2012

# Home Automation via XBEE modules

# Introduction :-

This project is a Home Automation application employing XBEE wireless transceiver modules. Through XBEE modules, we are able to send the signals from a Personal Computer to the AVR ATmega16 microcontroller wirelessly using its UART module. We just have to connect an XBEE module to PC via USB port and it will be able to send & receive data to & from the other XBEE module. In this project default XBEE settings will be used provided both XBEEs have same address, so they can communicate with each other. This project has total 5 forms including 2 main GUI forms. The first GUI controls DC & Stepper motor’s parameters such as speed, rotation and step angle. The 2nd GUI enables user to control Lights, write on a 16x2 LCD and monitor the temperature as well as set alarm temperature as per desire. All the programming is done in C# using visual studio windows forms and with the devcomponents dotnetbar & NI Measurement Studio tools.

# Background:-

Before using the XBEE modules, it is necessary to have sound knowledge about them. So, the basic info about the hardware used in this project is given below :

# XBEE Modules :-

XBee is the brand name from Digi International for a family of form factor compatible radio modules. These radios use a microchip made by Freescale to provide simple, standards-based point-to-point communications, as well as a proprietary implementation of mesh networking.

These modules are capable of transmitting and receiving data according to protocol IEEE 802.15.4, which is for wireless communication. XBEE radios provide a 30m range of successful data transmission & reception and the range can be 100m in the line-of-sight. XBEE modules provide a UART connection which is used for data in-out.

We have used a XBEE Series 1 transceiver module pair, each transceiver is capable of transmitting and receiving data via UART module wirelessly. The XBEE module is responsible for the transmission and reception of all signals generated by the PC and links them to the Microcontroller via RX and TX pins.

# DC Motor:-

In this project, a 5V dc motor is used for moving clockwise and anti-clockwise and its various parameters e.g. direction and speed (PWM) are varied by the operator.

# Stepper Motor:-

In this project, a 5V stepper motor is used and its direction, rotations and step angle are controlled as per requirement by the user.

# AVR ATmega16 microcontroller:-

In this project, a AVR ATmega16 is used for controlling dc motor, stepper motor and for sending and receiving data via XBEE module. The microcontroller also controls a 16x2 Text LCD and a Bargraph LED.

# Text LCD:-

In this project, a 16x2 Text LCD is used, for typing data and displaying on LCD. It is controlled by the microcontroller and is interfaced via open-source Peter Fleury LCD library.

# LM35 Temperature Sensor:-

A lm35 temperature sensor is used to send the current room temperature to the Microcontroller and gets displayed on the thermometer display in the GUI.

# Bargraph LED:-

In this project, a 10 LED bargraph is used to switch the lights desired by the operator.

# Power Supply Adapter:-

In this project, 2 variable power supply adapters are being used. One adapter is responsible to supply 5V to XBEE modules, Text LCD and ATmega16. The other adapter is used to run DC Motor and Stepper Motor.

# Veroboard:-

A large veroboard is used for mounting all the above mentioned components.

# The Code:-

We have made 5 forms for our project “Home Automation via XBEE”. Let us go through each of them one-by-one.

# The Welcome Form:-

As a welcome form, we have created a flash form which appears at the centre of the screen for 4 seconds and shows the Title of the Project and names of project members, then disappears. Here is a picture of it. Throughout our code, this form is named as Form2. Since, our project involves UART module so we have used USB-to-serial cable to interface XBEE module with the PC and COM5 is being used as serial port.

To make this form in my project namespace serialporthypterm, I selected a Windows form and disabled its maximize, minimize buttons and set its Start position property as Center Screen while Control Box property is also set as false. The pictures are added via Picture Box tool. A timer is also used to make sure the form runs for 4 seconds and then vanishes. Timer is initially enabled via the Properties box. In timer1_Tick event I have set the above mentioned properties in the code as:

namespace serialporthypterm

{

public partial class Form2 : Form

{

public Form2()

{

InitializeComponent();

}

// When the timer1 ticks 4000 times i.e. 4s, then welcome form vanishes,

// and wizard form i.e. Form4 appears.

private void timer1_Tick(object sender, EventArgs e)

{

this.Visible = false;           // form invisible

FORM4 fo = new FORM4();         //  constructor of Form4

fo.Show();			   // show Form4

timer1.Enabled = false;         // timer disabled

}

}

}

After 4 seconds, this form is vanished.

# The Wizard Form:-

The welcome form, after 4 seconds initializes a constructor which is bound to show the Form4 which is the Wizard form in our Project. This form shows a bit of detail about the project and also asks what GUI a user wants to select. Here is how it looks :

The above form is used from an application Devcomponents Dotnetbar which has a built-in form in the toolbar menu for selecting Wizard form. I have placed the XBEE hardware picture in the picture box and have written the details of project in it.  When user presses cancel button, the whole project closes, when next button pressed then next page appears as :

The user selects the desired GUI and proceeds. Hence this form disappears and another form i.e. Motor control or Temperature sensor forms appears as per user’s desire. The code is given below :

namespace serialporthypterm

{

public partial class FORM4 : Form

{

public FORM4()

{

InitializeComponent();

}

// disable textbox on loading FORM4

private void FORM4_Load(object sender, EventArgs e)

{

textBox1.Visible = false;

}

//if cancel button is pressed, then whole app closes down

private void wizard1_CancelButtonClick(object sender, CancelEventArgs e)

{

Close();

Environment.Exit(1);

}

// On press of next button, 2nd page of wizard form appers,

// asking the GUI to be selected

private void wizard1_NextButtonClick(object sender, CancelEventArgs e)

{

textBox1.Visible = true;

radioButton1.Checked = false;

radioButton2.Checked = false;

wizardPage2.Select();

wizardPage2.PageTitle = "Select the GUI you want to use”; // title of wizard form

wizardPage2.PageDescription = null;

// disable finish button

wizardPage2.FinishButtonEnabled = DevComponents.DotNetBar.eWizardButtonState.False;

// disable next button

wizardPage2.NextButtonEnabled = DevComponents.DotNetBar.eWizardButtonState.False;

}

// On selecting radio button1, Motor control GUI is displayed

// and Finish button is enabled

private void radioButton1_CheckedChanged(object sender, EventArgs e)

{

textBox1.Text = "Enables you to control DC motor

wizardPage2.FinishButtonEnabled=DevComponents.DotNetBar.eWizardButtonState.True;

// On selecting radio button2, Temperature and Light control

// GUI is selected and Finish button is enabled

private void radioButton2_CheckedChanged(object sender, EventArgs e)

{

textBox1.Text = "Enables you to monitor temperature, control lights and write on LCD";

wizardPage2.FinishButtonEnabled = DevComponents.DotNetBar.eWizardButtonState.True;

}

// The backbutton takes wizard page back to 1st page from 2nd page

private void wizard1_BackButtonClick(object sender, CancelEventArgs e)

{

textBox1.Visible = false;

wizardPage2.FinishButtonEnabled = DevComponents.DotNetBar.eWizardButtonState.False;

}

// When the finish button is clicked, form vanishes and selects

// either the form1 or form3 selected via the radio button by user

private void wizard1_FinishButtonClick(object sender, CancelEventArgs e)

{

// show motor control form

this.Visible = false;

if (radioButton1.Checked == true)

{

Form3 hee = new Form3();

hee.Show();

}

// show temperature sensor form

else if (radioButton2.Checked == true)

{

Form1 cs = new Form1();

cs.Show();

}

else { }

}

}

}

# Motor Control form:-

In this form, the user has the liberty to choose either the Stepper motor or the DC motor and vary its parameters. Here is its picture :

The code required to make this form is as follows:

using System;

using System.Collections.Generic;

using System.ComponentModel;

using System.Data;

using System.Drawing;

using System.Linq;

using System.Text;

using System.Windows.Forms;

namespace serialporthypterm

{

public partial class Form3 : Form

{

Int32 c, g, b, z, f, q, mn;

char[] buff = new char[1];

char[] w = new char[1];

string recvd;

public Form3()

{

InitializeComponent();

}

/* main dcmotor control part */

private void DCMotor(int dir, int spd)

{

switch (dir)

{

case 0:

//for speed 25% clock wise

if (spd == 25)

{

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

buff[0] = 'e';

serialPort1.Write(buff, 0, 1);

}

//for speed 50% clock wise

else if (spd == 50)

{

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

buff[0] = 'f';

serialPort1.Write(buff, 0, 1);

}

//for speed 75% clock wise

else if (spd == 75)

{

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

buff[0] = 'g';

serialPort1.Write(buff, 0, 1);

}

//for speed 100% clock wise

else if (spd == 100)

{

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

buff[0] = 'h';

serialPort1.Write(buff, 0, 1);

}

break;

case 1:

/* for speed 25% anti-clock wise */

if (spd == 25)

{

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

buff[0] = 'a';

serialPort1.Write(buff, 0, 1);

}

/* for speed 50% anti-clock wise */

else if (spd == 50)

{

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

buff[0] = 'b';

serialPort1.Write(buff, 0, 1);

}

/* for speed 75% anti-clock wise */

else if (spd == 75)

{

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

buff[0] = 'c';

serialPort1.Write(buff, 0, 1);

}

/* for speed 100% anti-clock wise */

else if (spd == 100)

{

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

}

break;

}

}

/* Stepper Motor control : continuous rotation panel */

public void continuousrotation(char[] e, int f)

{

switch (f)

{

/* sending no. of rotations with clock wise rotation */

case 0:

buff[0] = 'a';

serialPort1.Write(buff, 0, 1);

serialPort1.Write(e, 0, 1);

break;

/* sending no. of rotations with anti-clock wise rotation */

case 1:

buff[0] = 'c';

serialPort1.Write(buff, 0, 1);

serialPort1.Write(e, 0, 1);

break;

}

}

/* Stepper Motor Control: Step rotation part */

public void stepcontrol(int s, int cc)

{

switch (cc)

{

/* step rotation:45 degree,clockwise */

case 0:

if (s == 45)

{

buff[0] = 's';

serialPort1.Write(buff, 0, 1);

buff[0] = 'e';

serialPort1.Write(buff, 0, 1);

}

/* step rotation:90 degree,clockwise */

else if (s == 90)

{

buff[0] = 's';

serialPort1.Write(buff, 0, 1);

buff[0] = 'f';

serialPort1.Write(buff, 0, 1);

}

/* step rotation:180 degree,clockwise */

else if (s == 180)

{

buff[0] = 's';

serialPort1.Write(buff, 0, 1);

buff[0] = 'g';

serialPort1.Write(buff, 0, 1);

}

//step rotation:360 degree,clockwise

else if (s == 360)

{

buff[0] = 's';

serialPort1.Write(buff, 0, 1);

buff[0] = 'h';

serialPort1.Write(buff, 0, 1);

}

break;

case 1:

/* step rotation:45 degrees,anti-clockwise */

if (s == 45)

{

buff[0] = 's';

serialPort1.Write(buff, 0, 1);

buff[0] = 'a';

serialPort1.Write(buff, 0, 1);

}

/* step rotation:90 degrees,anti-clockwise */

else if (s == 90)

{

buff[0] = 's';

serialPort1.Write(buff, 0, 1);

buff[0] = 'b';

serialPort1.Write(buff, 0, 1);

}

/* step rotation:180 degrees,anti-clockwise */

else if (s == 180)

{

buff[0] = 's';

serialPort1.Write(buff, 0, 1);

buff[0] = 'c';

serialPort1.Write(buff, 0, 1);

}

/* step rotation:360 degrees,anti-clockwise */

else if (s == 360)

{

buff[0] = 's';

serialPort1.Write(buff, 0, 1);

buff[0] = 'd';

serialPort1.Write(buff, 0, 1);

}

break;

}

}

/* selecting continuous rotation part of stepper motor control */

private void radioButton2_CheckedChanged(object sender, EventArgs e)

{

groupBox1.Enabled = false;

groupBox2.Enabled = true;

}

/* entered in the stepper control portion & choosing the step rotation panel */

private void radioButton1_CheckedChanged(object sender, EventArgs e)

{

groupBox2.Enabled = false;

groupBox1.Enabled = true;

}

/* entering no. of rotations */

private void textBox1_Click(object sender, EventArgs e)

{

textBox1.Clear();

}

/* choosing the step rotation angle */

private void comboBox1_SelectedIndexChanged(object sender, EventArgs e)

{

g = Int32.Parse(comboBox1.Text);

}

/* choosing clockwise rotation in step rotation part of stepper motor control */

private void radioButton3_CheckedChanged(object sender, EventArgs e)

{

c = 0;

}

/* choosing anti-clockwise rotation in step rotation part of stepper motor control */

private void radioButton4_CheckedChanged(object sender, EventArgs e)

{

c = 1;

}

/* choosing step rotation panel in Stepper Motor control */

private void radioButton1_Click(object sender, EventArgs e)

{

serialPort1.Write("0");

z = 0;

}

/* choosing continuous rotation panel in Stepper Motor control */

private void radioButton2_Click(object sender, EventArgs e)

{

serialPort1.Write("1");

z = 1;

}

/* choosing clockwise rotation in continuous rotation part of stepper motor control */

private void radioButton5_CheckedChanged(object sender, EventArgs e)

{

b = 0;

}

/* choosing anti-clockwise rotation in continuous rotation part of stepper motor control */

private void radioButton6_CheckedChanged(object sender, EventArgs e)

{

b = 1;

}

/* calling main function to run DC motor */

private void button4_Click(object sender, EventArgs e)

{

DCMotor(f, q);

}

/* choosing clockwise rotation in DC motor control */

private void radioButton7_CheckedChanged(object sender, EventArgs e)

{

f = 0;

}

/* choosing anti-clockwise rotation in DC motor control */

private void radioButton8_CheckedChanged(object sender, EventArgs e)

{

f = 1;

}

/* selecting speed(duty cycle) for running dc  motor */

private void comboBox3_SelectedIndexChanged(object sender, EventArgs e)

{

q = Int32.Parse(comboBox3.Text);

}

/* selecting stepper Motor control and its image appears */

private void radioButton9_CheckedChanged(object sender, EventArgs e)

{

mn = 0;

groupBox6.Enabled = true;

groupBox3.Enabled = false;

this.reflectionImage1.Image = Image.FromFile(@"C:\Documents and Settings\evo\My Documents\My Pictures\stepper motor.jpg");

}

/* selecting DC Motor control and its image appears */

private void radioButton10_CheckedChanged(object sender, EventArgs e)

{

mn = 1;

groupBox3.Enabled = true;

groupBox6.Enabled = false;

this.reflectionImage1.Image = Image.FromFile(@"C:\Documents and Settings\evo\My Documents\My Pictures\dc motor.jpg");

}

/* FORM CHANGE if selected from File menu by user */

private void changeFormToolStripMenuItem_Click(object sender, EventArgs e)

{

serialPort1.Write("=");         // tell AVR microcontroller to change FORM

serialPort1.Close();

this.Hide();

FORM4 hoo = new FORM4();

hoo.Show();                     // display wizard form

}

/* Exit the whole program */

private void exitToolStripMenuItem_Click(object sender, EventArgs e)

{

serialPort1.Close();

Close();

Environment.Exit(1);

}

/* Open the About box */

private void projectInfoToolStripMenuItem_Click(object sender, EventArgs e)

{

AboutBox1 loo = new AboutBox1();

loo.Show();

}

/* Initializing serial port communication at COM5, 9600 baud rate */

private void buttonX1_Click(object sender, EventArgs e)

{

serialPort1.PortName = "COM5";

serialPort1.BaudRate = 9600;

serialPort1.Open();

if (serialPort1.IsOpen)

{

radioButton1.Enabled = true;

radioButton2.Enabled = true;

buttonX1.Enabled = false;

groupBox1.Enabled = false;

groupBox2.Enabled = false;

}

}

/* rotating either step rotation part or continuous part */

private void buttonX2_Click(object sender, EventArgs e)

{

switch (z)

{

case 0:

stepcontrol(g, c);

break;

case 1:

w[0] = char.Parse(textBox1.Text);

continuousrotation(w, b);

break;

}

}

/* calling main function to run DC motor */

private void buttonX3_Click(object sender, EventArgs e)

{

DCMotor(f, q);

}

/* Close serial port on exiting program */

private void Form3_FormClosing(object sender, FormClosingEventArgs e)

{

serialPort1.Close();

Environment.Exit(1);

}

/* Tell microcontroller that DC Motor form is active */

private void Form3_Activated(object sender, EventArgs e)

{

if (serialPort1.IsOpen)

{

serialPort1.Write("1");

}

else

{

serialPort1.Open();

serialPort1.Write("1");

}

}

}

}

# Temperature & Lights control form :-

In this form, we can observe real time temperature, room temperature and control Lights as well as write text on a 16x2 LCD. The GUI looks like this :

The thermometer is drawn via the NI Measurement Studio toolbar, which indicates current temperature and displays the room temperature as well as current temperature in the textbox. Also one can set an alarm temperature. When the LM35 temperature sensor output exceeds the set temperature then a buzzer beeps sounding emergency alarm.

# Visual Studio code :-

Here is the visual studio code to do the necessary operations and display on GUI:

using System;

using System.Collections.Generic;

using System.ComponentModel;

using System.Data;

using System.Drawing;

using System.Linq;

using System.Text;

using System.Media;

using System.Windows.Forms;

using System.Windows.Forms.VisualStyles;

using System.Threading;

namespace serialporthypterm

{

public partial class Form1 : Form

{

double t = 0, global = 0;

string handle,current;

int a = 0, b = 0, c = 0, d = 0, i = 0, f = 0, g = 0, h = 0,alarmtemp=100;

public Form1()

{

InitializeComponent();

}

/* Open serial port and select form 1 on form load */

private void Form1_Load(object sender, EventArgs e)

{

/* Check if serial port is open, and select GUI2 on AVR */

if (serialPort1.IsOpen)

{

serialPort1.Write("2");

}

else

{

serialPort1.Open();

serialPort1.Write("2");

}

}

/* On serial port receive event take temperature readings per second and */

/* display it on thermometer and textbox */

private void serialPort1_DataReceived(object sender, System.IO.Ports.SerialDataReceivedEventArgs e)

{

/* update temperature reading per second */

timer1.Start();

if (timer1.Interval == 1000)

{

/* read room temperature once only */

if (global == 0)

{

handle = serialPort1.ReadExisting();

textBox4.Text = handle;			// display on room temperature box

global++;

}

/* now keep reading current temperature */

else

{

current = serialPort1.ReadExisting();

textBox2.Text = current;			//display on current temperature box

t = Convert.ToDouble(current);

thermometer1.value = t;

/* give various colors to thermometer on temperature ranges */

if (t >= 10 && t < 20)

{

thermometer1.FillColor = Color.Red;

thermometer1.BackColor = Color.Blue;

}

if (t >= 20 && t < 30)

{

thermometer1.FillColor = Color.Red;

thermometer1.BackColor = Color.Aqua;

}

if (t >= 30 && t < 40)

{

thermometer1.FillColor = Color.Red;

thermometer1.BackColor = Color.Yellow;

}

if (t >= 40 && t < 100)

{

thermometer1.FillColor = Color.Red;

thermometer1.BackColor = Color.Red;

}

/* if temperature exceeds 100Celsius then show message */

if (t >= 100)

{

MessageBox.Show("Temperature out of range");

}

/* if temperature exceeds alarm temperature then raise alarm */

if (t >= alarmtemp)

{

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\smokealarm.wav");

sound.Play();

}

else { }

current = null;

}

}

}

/* Transfer characters written on textbox to LCD */

private void textBox1_TextChanged(object sender, EventArgs e)

{

serialPort1.Write("[");

serialPort1.Write(textBox1.Text);

}

/* LCD Clear on button press */

private void buttonX1_Click(object sender, EventArgs e)

{

serialPort1.Write("[");

}

/* Textbox clear on button press */

private void buttonX2_Click(object sender, EventArgs e)

{

textBox1.Clear();

}

/* Led 1 status toggle if button 1 is pressed with a beep sound */

private void led9_Click(object sender, EventArgs e)

{

if (a==0)

{

a = 1;

serialPort1.Write("1");

led9.Value=true;

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

else

{

a = 0;

serialPort1.Write("1");

led9.Value = false;

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware remove.wav");

sound.Play();

}

}

/* Led 2 status toggle if button 2 is pressed with a beep sound */

private void led10_Click(object sender, EventArgs e)

{

if (b == 0)

{

led10.Value = true;

b = 1;

serialPort1.Write("2");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

else

{

b = 0;

led10.Value = false;

serialPort1.Write("2");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware remove.wav");

sound.Play();

}

}

/* Led 3 status toggle if button 3 is pressed with a beep sound */

private void led11_Click(object sender, EventArgs e)

{

if (c == 0)

{

led11.Value = true;

c = 1;

serialPort1.Write("3");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

else

{

c = 0;

led11.Value = false;

serialPort1.Write("3");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware remove.wav");

sound.Play();

}

}

/* Led 4 status toggle if button 4 is pressed with a beep sound */

private void led12_Click(object sender, EventArgs e)

{

if (d == 0)

{

led12.Value = true;

d = 1;

serialPort1.Write("4");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

else

{

d = 0;

led12.Value = false;

serialPort1.Write("4");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware remove.wav");

sound.Play();

}

}

/* Led 5 status toggle if button 5 is pressed with a beep sound */

private void led13_Click(object sender, EventArgs e)

{

if (f == 0)

{

led13.Value = true;

f = 1;

serialPort1.Write("5");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

else

{

f = 0;

led13.Value = false;

serialPort1.Write("5");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware remove.wav");

sound.Play();

}

}

/* Led 6 status toggle if button 6 is pressed with a beep sound */

private void led14_Click(object sender, EventArgs e)

{

if (g == 0)

{

led14.Value = true;

g = 1;

serialPort1.Write("6");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

else

{

g = 0;

led14.Value = false;

serialPort1.Write("6");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware remove.wav")

sound.Play();

}

}

/* Led 7 status toggle if button 7 is pressed with a beep sound */

private void led15_Click(object sender, EventArgs e)

{

if (h == 0)

{

led15.Value = true;

h = 1;

serialPort1.Write("7");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

else

{

h = 0;

led15.Value = false;

serialPort1.Write("7");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware remove.wav");

sound.Play();

}

}

/* Led 8 status toggle if button 8 is pressed with a beep sound */

private void led16_Click(object sender, EventArgs e)

{

if (i == 0)

{

led16.Value = true;

i = 1;

serialPort1.Write("8");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

else

{

i = 0;

led16.Value = false;

serialPort1.Write("8");

System.Media.SoundPlayer sound = new System.Media.SoundPlayer(@"C:\WINDOWS\Media\windows xp hardware insert.wav");

sound.Play();

}

}

/* sets the alarm temperature in textbox4 */

private void buttonX4_Click(object sender, EventArgs e)

{

try

{

alarmtemp = Convert.ToInt16(textBox3.Text);

}

catch (FormatException)

{

}

}

/* Show GUI selection menu and change FORM */

private void changeGUIToolStripMenuItem_Click(object sender, EventArgs e)

{

serialPort1.Write("=");             // tell AVR to change form

timer1.Start();

while (timer1.Interval == 200) ; // wait for 200ms

serialPort1.Close();

this.Hide();

FORM4 he = new FORM4();

he.Show();

}

/* Exit the program */

private void exitToolStripMenuItem_Click(object sender, EventArgs e)

{

serialPort1.Close();

Close();

Environment.Exit(1);

}

/* Show the Project details in About box */

private void projectInfoToolStripMenuItem_Click(object sender, EventArgs e)

{

AboutBox1 lo = new AboutBox1();

lo.Show();

}

/* Close the whole program and serial port */

private void buttonX3_Click(object sender, EventArgs e)

{

/* close the serial PORT and previously ON leds */

if (a != 0)

serialPort1.Write("1");     // close LED1

if (b != 0)

serialPort1.Write("2");     // close LED2

if (c != 0)

serialPort1.Write("3");     // close LED3

if (d != 0)

serialPort1.Write("4");     // close LED4

if (f != 0)

serialPort1.Write("5");     // close LED5

if (g != 0)

serialPort1.Write("6");     // close LED6

if (h != 0)                                                                                              					    serialPort1.Write("7");     // close LED7

if (i != 0)

serialPort1.Write("8");     // close LED8

else { }

/* Clear the LCD screen */

serialPort1.Write("[");

/* Close serial Port */

serialPort1.Close();

Environment.Exit(1);            // terminates the whole program

Close();

}

/* open serial port and select form 1 every time this form is selected */

private void Form1_Activated(object sender, EventArgs e)

{

if (serialPort1.IsOpen)

{

serialPort1.Write("2");

}

else

{

serialPort1.Open();

serialPort1.Write("2");

}

}

}

}

# About Box :-

In this form, we show description about the project. Here is the code :

namespace serialporthypterm

{

partial class AboutBox1 : Form

{

public AboutBox1()

{

InitializeComponent();

this.Text = String.Format("About {0}", AssemblyTitle);

this.labelProductName.Text = AssemblyProduct;

this.labelVersion.Text = String.Format("Version {0}", AssemblyVersion);

this.labelCopyright.Text = AssemblyCopyright;

this.labelCompanyName.Text = AssemblyCompany;

this.textBoxDescription.Text = AssemblyDescription;

}

#region Assembly Attribute Accessors

public string AssemblyTitle

{

get

{

object[] attributes = Assembly.GetExecutingAssembly().GetCustomAttributes(typeof(AssemblyTitleAttribute), false);

if (attributes.Length > 0)

{

AssemblyTitleAttribute titleAttribute = (AssemblyTitleAttribute)attributes[0];

if (titleAttribute.Title != "")

{

return titleAttribute.Title;

}

}

return System.IO.Path.GetFileNameWithoutExtension(Assembly.GetExecutingAssembly().CodeBase);

}

}

public string AssemblyVersion

{

get

{

return Assembly.GetExecutingAssembly().GetName().Version.ToString();

}

}

public string AssemblyDescription

{

get

{

object[] attributes = Assembly.GetExecutingAssembly().GetCustomAttributes(typeof(AssemblyDescriptionAttribute), false);

if (attributes.Length == 0)

{

return "";

}

return ((AssemblyDescriptionAttribute)attributes[0]).Description;

}

}

public string AssemblyProduct

{

get

{

object[] attributes = Assembly.GetExecutingAssembly().GetCustomAttributes(typeof(AssemblyProductAttribute), false);

if (attributes.Length == 0)

{

return "";

}

return ((AssemblyProductAttribute)attributes[0]).Product;

}

}

public string AssemblyCopyright

{

get

{

object[] attributes = Assembly.GetExecutingAssembly().GetCustomAttributes(typeof(AssemblyCopyrightAttribute), false);

if (attributes.Length == 0)

{

return "";

}

return ((AssemblyCopyrightAttribute)attributes[0]).Copyright;

}

}

public string AssemblyCompany

{

get

{

object[] attributes = Assembly.GetExecutingAssembly().GetCustomAttributes(typeof(AssemblyCompanyAttribute), false);

if (attributes.Length == 0)

{

return "";

}

return ((AssemblyCompanyAttribute)attributes[0]).Company;

}

}

#endregion

/* Hide this form on clicking OK button */

private void okButton_Click(object sender, EventArgs e)

{

this.Hide();

}

/* Set description text, product name, company name and version labels */

private void AboutBox1_Load(object sender, EventArgs e)

{

textBoxDescription.Text = "This project has been created by Shayan Ali Bhatti & Shahbaz Hasan of 6th Semester. The program is an application of Home Automation via XBEE modules interfaced with AVR ATMEGA 16. It has 2 GUI forms. The 1st GUI's purpose is to observe and change the various parameters such as the PWM(Speed) and ROTATION of a DC Motor. Also, a Stepper Motor's rotations and step angle can be varied as per requirement.The 2nd GUI's purpose is to get and display the temperature sensed by LM35, control Lights, and type on a 16x2 LCD wirelessly via XBEE modules.";

labelProductName.Text = "Home Automation via XBEE";

labelCompanyName.Text = "Students";

labelVersion.Text = "Version : 1.00";

}

}

}

# Things I learnt during this project :-

By doing this project, we became able to use serial port and understood the vast scope of C# whilst working with the hardware. We also concluded that by using Devcomponents Dotnetbar application and NI Measurement Studio tools, one can easily make a very good GUI which is easy to make & use. Also, whilst using LM35 temperature sensor, proper measures should be taken to filter noise, otherwise temperature will keep changing producing a bad output.

# Final word:-

We, Shayan Ali Bhatti & Shahbaz Hasan are very much thankful to Almighty Allah and our teacher Sir Sajid Hussain, who kept guiding us with the best of his knowledge about C# programming and helped us to develop our programming skills. We hope that this project will be helpful to the members of Code Project website.